# Input  and output formats for spin transport simulations

This list is not necessarily authoritative, it just captures our current collective understanding.

## PPS-Sim

Input: x, x’, y, y’, E

Output: x, x’, y, y’, E, sx, sy, sz (defined wrt beam direction), z (from ‘particle source’ – exact meaning of which is user defined), t (arbitrary zero)

## BMAD

Input: user definable (bunch charge, time and z relative to head of first bunch, bunch start and end)

Internal: (canonical phase space co-ords), x, px,y,  py z, pz + spinors

Output: as for input

## Zgoubi

Input: x, x’, y, y’, delta_p/p, delta_s (wrt centre of bunch), sx, sy, sz (wrt design orbit) (single particle tracking)

Output: as for input

## Slicktrack

Input:  optical code + for beam sigma_x, sigma_y, sigma_z (MC)

Output:  user definable

MAD-X / PTC (when being used for single particle tracking in PTC if this is possible)

Input: x, px, y, py, E,  + beam parameters (we  think....)

Output: as above

## CAIN

Input:  x, y, z (m), E (eV), px, py, pz (eV/c), sx, sy, sz (wrt design orbit), bunch charge

Output: same as above

## GP++

Input: x, px, y, py, z, pz, sx, sy (possibly?)

Output: E, x, y, z, vx, vy, sx, sy, sz

We identified three types of “interface”:

- Lattice conversion tools

- Online frameworks  i.e. a common wrapper for two or more simulation codes that handles execution and i/o between the codes

- Offline files

# Lattice Conversion Tools

We are aware of these conversion tools:

Mad-X Slicktrack

Mad-XZgoubi

Mad-8Slicktrack

Mad-8Zgoubi

Accelerator Markup Language & UAP (D. Sagan and A. Wolski) with conversion between .xsif, mad-8, mad-X, bmad and sad.

# Online Frameworks

- Python

- Matlab

- Octave (Tony)

The recommendation of the workshop was to use Python.

NB Combination of Geant4 with lattice transport is available through BDSIM and G4 Beamline.

# Offline Files

Our concept for this is to adopt a standardised syntax for files being exchanges between spin transport codes.  Work in progress!

The file will consist of a meta-data header  followed by particle data (one particle per line).

File format: ASCII

Meta data from input appended to output, except in tracking many turns around a periodic structure...

Meta data tags: Time-stamp, Program ID, Version, Iteration,  Simulation Complete flag, OS, dependency version,  comments, synopsis of input particle ensemble (?)

Exchange file syntax:

Particle ID (PDG standard), E , x, y, z, sx, sy, sz, x’, y’,

Joint University Accelerator School (JUAS) units / frame definitions